Network Design Project.

Personal Information

- Name: Hazem Hamdi Mahmoud Abdelqader

- University: Nahda University (NUB)

- Faculty: Faculty of Engineering

- Department: Communications and Electronics Engineering

Contact Details

- Phone: +20 106 238 6571

- Email: hazemhamdypersonal@gmail.com

- GitHub: github.com/Hazem-Hamd

| Vlan | Department | N of devices | IP | Subnet Mask | range |
| --- | --- | --- | --- | --- | --- |
| 3 | Marketing | 50 | 192.168.0.0 | /26 | 0 – 63 |
| 4 | Accounting | 25 | 192.168.0.64 | /27 | 64 - 95 |
| 5 | Purchasing | 13 | 192.168.0.96 | /28 | 96 - 111 |
| 6 | HR | 2 | 192.168.0.112 | /29 | 112 - 119 |
| 7 | IT | 1 | 192.168.0.120 | /30 | 120-123 |
| - | Mult-Rout | 2 | 192.168.0.124 | /30 | 124-127 |

| Vlan | Department | N of devices | IP | Subnet Mask | range |
| --- | --- | --- | --- | --- | --- |
| 8 | Marketing | 50 | 192.168.0.128 | /26 | 129-191 |
| 9 | Accounting | 25 | 192.168.0.192 | /27 | 193-223 |
| 10 | Purchasing | 13 | 192.168.0.224 | /28 | 225-239 |
| 11 | HR | 2 | 192.168.0.240 | /29 | 241-247 |
| 12 | IT | 1 | 192.168.0.248 | /30 | 249-251 |
| - | Mult-Rout | 2 | 192.168.0.252 | /30 | 253-254 |

| Vlan | Department | N of devices | IP | Subnet Mask | range |
| --- | --- | --- | --- | --- | --- |
| 13 | Marketing | 50 | 192.168.1.0 | /26 | 0 – 63 |
| 14 | Accounting | 25 | 192.168.1.64 | /27 | 64 – 95 |
| 15 | Purchasing | 13 | 192.168.1.96 | /28 | 96 – 111 |
| 16 | HR | 2 | 192.168.1.112 | /29 | 112 – 119 |
| 17 | IT | 1 | 192.168.1.120 | /30 | 120-123 |
| - | Mult-Rout | 2 | 192.168.1.124 | /30 | 124-127 |
| - | Servers Farm | 3 | 192.168.1.128 | /29 | 128-135 |
| - | RB1-ISP | 2 | 192.168.1.136 | /30 | 136-139 |
| - | WAN (RB3-RB2) | 2 | 192.168.1.140 | /30 | 141 – 142 |
| - | WAN (RB2-RB1) | 2 | 192.168.1.144 | /30 | 145 – 146 |
| - | RB2-ISP | 2 | 192.168.1.148 | /30 | 149-151 |
| - | RB3-Isp | 2 | 192.168.1.152 | /30 | 153-155 |

## 3. IP Addressing Strategy Variable Length Subnet Masking (VLSM)
The main network that I want to divide from  192.168.0.0/24  Class C
192.168.0.0
255.255.255.0

| Marketing | 50 |
| --- | --- |
| Accounting | 25 |
| Purchasing | 13 |
| HR | 2 |
| IT | 1 |
| Mult-Rout | 2 |

First ….
We need to round the number of devices to the nearest number that I can represent as a binary number to calculate the number of bits.
Log2(N of devices) =  n  The closest number to the number of devices I can represent in binary.
Log2(50)=5.643 ≈ 6 = n  Number of Bits That needed to include the 50 devices N of Devices 26 = 64.
second….
in the network to calculate.
The Number of Hosts  2total number of zeroes
We need 64 host to calculate the number of zeroes   
Log2(64)=6  zeroes 
192.168.0.0
255.255.255.11000000
The Number of Network  2total number of ones     22 = 4  Network 
IF we Used a Fixed length submitting mask (FLSM )
…..
192.168.0.0/26
192.168.0.64/26
192.168.0.128/26
192.168.0.192/26


Which will waste a lot of Ip-address

Can fix this use Variable Length Subnet Masking (VLSM) .
take the second network and modify it.
192.168.0.64/26
255.255.255.11000000
n  5                           N of Devices  25 = 32
Number of zeroes = 5
Number of ones = 8 – 5 = 3

192.168.0.64
255.255.255.11100000

The network will become  192.168.0.64/27 
Apply this to all other network make sure that we following the rule of non-blocking size.


the rule of non-blocking size  Need the number of host represent in decimal / the decimal number located at the last byte to be an integer
64/32 = 2 ✅

DHCP Implementation
 Automation: The DHCP server automatically assigns IPs from a pool whenever a device joins the network.

Mobility: Devices moving between VLANs (e.g., from IT to Guest) automatically receive the correct IP for their new subnet.

Conflict Prevention: The server tracks leased addresses, ensuring no two devices receive the same IP.

Configuration: Static IPs are manually assigned to router interfaces and SVIs (Switch Virtual Interfaces). The Ip helper-address 192.168.1.130 command is utilized to forward DHCP requests to the central server.

Code Example (On Router/Switch Interfaces):

Enable conf t ! --- VLAN N (Dept) interface vlan N ! The IP of the VLAN Gateway Ip address IP SUB ! THE HELPER COMMAND: Forwards DHCP requests to the central server Ip helper-address 192.168.1.130 exit

- 4. EtherChannel Implementation

EtherChannel bundles multiple physical cables into a single logical interface.

- Benefits: It provides high bandwidth by combining speeds (e.g., two 100Mbps links create a 200Mbps connection) and ensures redundancy if one cable is severed.

- Configuration: Physical ports are selected and assigned to a channel group using LACP in active mode. The newly created logical interface is then configured as a trunk to carry all VLAN traffic

Enable conf t ! 1. Select the physical ports to be bundled interface range fastEthernet 0/1-2 ! 2. Assign them to a channel group using LACP (mode active) channel-group 1 mode active no shutdown exit ! 3. Configure the new Logical Interface interface port-channel 1 ! Make it a Trunk to carry all VLAN traffic switchport mode trunk switchport trunk encapsulation dot1q ! (If required by the switch model) no shutdown exit

5: Port Security 

 Port Security memorizes the MAC Address (the unique hardware ID) of the first device that connects to the port.

If a stranger tries to plug in a different device (which has a different MAC address), the switch immediately shuts down the port and blocks access.

Enable conf t ! 1. Select the interface interface fastEthernet 0/1 ! 2. Set mode to Access switchport mode access ! 3. Enable Port Security switchport port-security ! 4. Manually assign the specific MAC Address (Static) ! (Replace this number with your real PC's MAC) switchport port-security mac-address 00E0.F725.337A ! 5. Set the violation mode to Shutdown switchport port-security violation shutdown exit show port-security interface fastEthernet 0/1

6. VTP Implementation (VLAN Trunking Protocol)

 In a large company with 20 switches, if you want to add a new "IT Dept" VLAN (VLAN 10), you normally have to log into all 20 switches and type the commands manually. This is slow and you might make mistakes.

VTP lets you create the VLANs once on a central switch (called the Server). Then, VTP automatically "broadcasts" these VLANs to all other switches (called Clients).

On the Main Switch (VTP Server)

Enable conf t! 1. Set the VTP Domain Name (Must be the same on all switches) vtp domain amazon ! 2. Set a Password (Optional but recommended for security) vtp password 123 ! 3. Set the mode to Server (This is usually the default) vtp mode server ! 4. Now, create your VLANs here vlan 10 name IT vlan 20 name HR vlan 30name Sales exit

On the Access Switches (VTP Clients).

Enable conf t ! 1. Match the Domain Name (Crucial!) vtp domain amazon ! 2. Match the Password vtp password 123 ! 3. Change mode to Client vtp mode client ! 4. Important: Ensure the link to the server is a TRUNK interface fastEthernet 0/1 ! (Assuming this connects to the Main Switch) switchport mode trunk exit

#### Verification

To check if the Client switch successfully downloaded the VLANs from the Server:

show vtp status

show vlan brief

7. Secure Remote Access (SSH)
We need to access the routers remotely
It encrypts the connection (turns data into unreadable code) so no one can steal the password.

Enable conf t ! 1. Change Hostname & Domain (Required for SSH keys) hostname RB1 ! (Or RB2, RB3 depending on the router) ip domain-name amazon.com ! 2. Generate Encryption Keys (The "Lock") ! When asked for the size, type 1024 crypto key generate rsa ! 3. Create the Admin Account (The "Key") ! Privilege 15 gives full admin rights immediately username IT1 privilege 15 secret 123 ! 4. Create a Security List (ACL) ! Only allow the IT Computer IP, block everyone else access-list 1 permit host 192.168.1.122 ! 5. Apply settings to the Virtual Lines (VTY) line vty 0 4 ! Use the local username/password we created login local ! Force SSH only (Disable Telnet) transport input ssh ! Apply the Security List (The Guard at the door) access-class 1 in exit

TO ACCESS ON ROUTER

ssh -l IT1 192.168.X.X

8- Routing
Without Routing: Router 1 knows about its own cables, but it doesn't know where the networks in Router 2 or Router 3 are. If you ping from Branch 1 to Branch 3, the packet gets lost.

Version 2 because it supports VLSM

enable conf t ! 1. Enter RIP configuration mode router rip ! 2. Switch to Version 2 (Crucial for subnets) version 2 ! 3. Disable Auto-Summary (Very Important!) ! If you forget this, the router will mix up the subnets (e.g., /26 will become /24) no auto-summary ! 4. Advertise the networks you know ! Since all our IPs start with 192.168.x.x, we can add these two lines: network 192.168.1.0network 192.168.0.0 exit

9: TFTP
copy the current configuration file (running-config) from the router to a central server designated as the TFTP Server.
If a router dies, you simply plug in a new empty router and download the backup file from the server.

enable ! 1. (Optional) Check connection to the server first ping 192.168.1.130 ! 2. The Command to upload the configuration copy running-config tftp: ! --- The Router will ask you questions (Interactive Mode) --- ! Question 1: Address or name of remote host? ! Answer: Type the Server IP 192.168.1.130 ! Question 2: Destination filename? [running-config]? ! Answer: Type a unique name for this router RB1-Backup RB1-Backup-Jan2026

Restore

enable ! 1. The restore command (Copy FROM tftp TO running-config) copy tftp: running-config ! --- The Router will ask you questions (Interactive Mode) --- ! Address or name of remote host ! Type the TFTP Server IP 192.168.1.131 Source filename ! Type the EXACT name of the backup file you saved earlier RB1-Config-Backup !Destination filename? [running-config] ! Just press ENTER (to accept the default)

10: Default Route to ISP

send any unknown packet to ISP

enable conf t ! --- Part 1: Create the path to the Internet --- ! The "0.0.0.0 0.0.0.0" means "Any IP address, Any Subnet Mask" ! Serial0/1/0 is the exit interface towards the ISP ip route 0.0.0.0 0.0.0.0 Serial0/1/0 ! --- Part 2: Share this path with other branches --- ! We go into the RIP settings router rip ! This command tells RIP: "Tell RB2 and RB3 that I am the way to the internet" default-information originate exit

11: Restricting Ping Access (ACL)
everyone can ping everyone. This causes unnecessary traffic and allows anyone to map your network.
use an Extended ACL (Access Control List). It works like a sophisticated security guard.
The Logic:

Rule 1: "If the sender is the IT Computer (192.168.1.122), let the Ping pass."

Rule 2: "If the packet is just a Reply (an answer to a ping), let it pass." (This is crucial, otherwise, the IT guy sends a ping, but the answer gets blocked on its way back!).

Rule 3: "Block any other Ping request from anyone else."

Rule 4: "Allow all other traffic (Internet, Email, Files) to pass normally."

Enable conf t ! 1. Create Extended Access List 101 ! Allow IT Admin to ping anyone access-list 101 permit icmp host 192.168.1.122 any ! 2. Allow "Replies" to return ! (If you skip this, the IT Admin will send a ping but never hear the answer) access-list 101 permit icmp any any echo-reply access-list 101 permit icmp any any unreachable ! 3. Block all other Ping requests access-list 101 deny icmp any any ! 4. IMPORTANT: Allow all other traffic (Web, etc.) ! If you forget this, you kill the entire internet for everyone! access-list 101 permit ip any any

Apply the ACL to Interfaces

interface vlan N ip access-group 101 in exit